[Facility Name]

Facility ID [Facility ID]

Toxics Emissions Inventory Plan

for

AB 2588

California Air Toxics “Hot Spots”

Information and Assessment Act of 1987

[Date]

Submitted to:

San Joaquin Valley Unified Air Pollution Control District

1990 E Gettysburg Avenue

Fresno, California 93726

Prepared by:

[Facility Name]

[Facility Street]

[Facility City State Zip]

Table of Contents

[To update the table of contents, hit F9 and update the entire table. Do not manually change the page numbers.]

1.	Introduction	1

2.	Facility Location & Description	1

3.	Permitted and PEER Emission Sources	4

3.1	EXAMPLE: External Combustion (Boilers, Dehydrators, Heaters, Dryers), Natural Gas and/or LPG (Propane)	4

3.2	EXAMPLE: Turbine IC Engines, Natural Gas and/or LPG (Propane)	6

3.3	EXAMPLE: Reciprocating IC Engines, Diesel	7

3.4	EXAMPLE: Reciprocating IC Engines, Natural Gas and/or LPG (Propane)	8

3.5	EXAMPLE: Permitted PM10 Sources: Source-Specific Heading [Change the heading title to appropriately describe the source.]	9

3.6	EXAMPLE: Permitted VOC Sources: Source-Specific Heading [Change the heading title to appropriately describe the source.]	11

3.7	EXAMPLE: Other Permitted Sources [Change the heading title to appropriately describe the source.]	12

4.	Other Permit-Exempt Equipment	13

4.1	EXAMPLE: [Other Permit-Exempt Equipment Type]	13

5.	Other Permit-Exempt Fugitive Emissions	14

5.1	EXAMPLE: Chlorinated Water Processes	14

5.2	EXAMPLE: [Other Permit-Exempt Fugitive Emissions]	15

6.	Source Testing	16

6.1	No Source Testing	16

6.2	Proposed Source Testing	16

Appendix A.	Facility Process Flow Diagram(s)	18

Appendix B.	Permit and PEER Unit Listing	19

Appendix C.	Toxic Emissions Profiles	20

Appendix D.	Safety Data Sheets	31

Appendix E.	Stack Parameters	32

Appendix F.	Facility Map with Receptor Locations	33

# Introduction

The Air Toxics "Hot Spots" Information and Assessment Act (AB 2588 or the “Act”) was enacted in September 1987.  Under the Act, stationary sources are required to report the types and quantities of certain toxic substances their facilities routinely release into the air.  AB 2588 is designed to provide information to state and local agencies and to the general public on the extent of airborne emissions from stationary sources and the potential public health impacts of those emissions.  The San Joaquin Valley Air Pollution Control District (the District) is mandated by the State to implement AB 2588.

On March 6, 2015, The State Office of Environmental Health Hazard Assessment (OEHHA) adopted changes to the Air Toxics Hot Spots Program Guidance Manual for the Preparation of Health Risk Assessments.  These revisions were designed to incorporate three technical support documents and to provide enhanced protection of children as required under state law (SB 25, Escutia, 1999).  Due to these recent changes, and the corresponding potential increases in calculated health risk, the District notified [Facility Name] that we must be re-evaluated under AB 2588.

Pursuant to the Air Toxics "Hot Spots" Information and Assessment Act of 1987, we have prepared a comprehensive, site-specific Toxics Emissions Inventory Plan.  The plan specifies in detail how [Facility Name] will inventory our facility's emissions of all toxic substances on the list of substances subject to the Act.  The plan provides a comprehensive and detailed description of the methods that will be used to quantify air releases or potential air releases of listed substances from all points of release.  The plan also includes quantification methods which result in accurate and comprehensive characterization of releases and comply with all applicable requirements of the “Hot Spots” regulation.

# Facility Location & Description

[Indicate the actual location of the facility including the street address. Use Universal Transverse Meridian (UTM) coordinates, a descriptive location, or Mount Diablo Base Meridian location (section, township, and range) if the street address is not available.

Delete examples that do not apply.]

Example (a): (With a street address.)

The facility is located at 1990 E Gettysburg in Fresno, CA.

Example (b): (With a Mount Diablo Base Meridian Location.)

The facility is located at the 31X oil and water treatment plant, within the SW/4 of Section 31, Township 29S, Range 21E.

Example (c): (With a descriptive location.)

The facility is located on the eastern side of 25th Avenue, approximately one mile south of State Route (SR) 198, in Kings County.

[Briefly describe the facility’s operations.]

[If there are multiple facilities in a single stationary source, please add a statement as follows: “Facility X, Y, and Z are part of a single stationary source.”]

Examples:

(a) This facility is a grain and commodity receiving, processing, and packing operation. Grain and commodities are received from trucks or railcars via underground receiving pits. Received products are transferred into storage silos or hoppers via fully enclosed conveyors.  Feed is manufactured by grinding, flaking, and pelleting operations.  Products can be mixed and bulk loaded or bagged for distribution.

(b) This facility is an agricultural operation with the primary function of growing crops and/or raising of fowl or animals. Equipment at this facility includes units needed for agricultural production, such as stationary IC engines powering agricultural irrigation well pumps.

(c) This facility is a gasoline dispensing facility. Gasoline is delivered to the storage tank(s) via a delivery vessel.  Gasoline is then dispensed from the storage tank(s) into motor vehicle tanks during vehicle refueling.

(d) This facility is a glass and plastic bottle coating operation. Glass or plastic bottles are decorated utilizing either spray or screen-printing equipment. The coatings are cured utilizing UV lights, electric heaters, or natural gas fired lehrs.

This plan will address emission sources present during the most current and complete inventory year – 2018. During the reporting year (2018), [Facility Name] identified the following sources of emissions, which must be evaluated pursuant to AB 2588:

[Verify the information in the table below. This should be a list of permitted emission sources and Permit-Exempt Equipment Registration (PEER) sources, including associated source descriptions, District IDs, and toxics device IDs for each emission source.]

Table 1.  Permitted and PEER Emission Sources

| Source Description | District ID | Toxics Device ID |
| --- | --- | --- |

[Fill out Table 2 below with non-fugitive permit-exempt sources. Note: all PEERs are already accounted for in Table 1 above. Refer to Appendix A and Appendix C of the CARB Emission Inventory Criteria and Guidelines for details regarding the potential sources to include. Please contact the District’s Hot Spots Team at (559) 230-6000 or at hotspots@valleyair.org with any questions you may have.

The toxics device IDs used for non-fugitive permit-exempt sources will start at 6001 and will continue sequentially (6001, 6002, 6003, etc.). Number each device separately. Refer to Section 4 below for more information regarding this category.]

Table 2. Non-Fugitive Permit-Exempt Sources

| Source Description | Toxics Device ID |
| --- | --- |
| Example: 24 BHP GASOLINE ENGINE | 6001 |
| Example: XXXXXX | 6002 |
|  | 6003 |
| [Add additional rows as necessary] | 6004 |

[Fill out Table 3 below with fugitive permit-exempt sources. Note: all PEERs are already accounted for in Table 1 above. Refer to Appendix A and Appendix C of the CARB Emission Inventory Criteria and Guidelines for details regarding the potential sources to include.

The toxics device IDs used for fugitive permit-exempt sources will start at 7001 and will continue sequentially (7001, 7002, 7003, etc.). Number each device separately. Refer to Section 5 below for more information regarding this category.]

Table 3. Fugitive Permit-Exempt Sources

| Source Description | Toxics Device ID |
| --- | --- |
| Example: CHLORINATED WATER PROCESSES | 7001 |
| Example: XXXXXX | 7002 |
|  | 7003 |
| [Add additional rows as necessary] | 7004 |

[Each inventory plan shall include a flow diagram consisting of a comprehensive schematic drawing of the process flows that affect the nature or quantity of emission of listed substances. If necessary, a reference document shall be attached to the drawing to include any information needed to fulfill the flow diagram requirements that cannot be included in the drawing. Flow diagram requirements are listed under Section VIB of the CARB Emission Inventory Criteria & Guidelines www.arb.ca.gov/ab2588/2588guid.htm. For information regarding trade secrets, please refer to Section VIC of the CARB Emission Inventory Criteria and Guidelines or the CA Health & Safety Code § 44346.]

A facility process flow diagram identifying each emission source is provided in Appendix A.

# Permitted and PEER Emission Sources

This section describes the methods used to identify and quantify toxic emissions from sources subject to District permit or permit-exempt equipment registration (PEER).  For permitted sources, the toxics device ID used is the same as the permit ID.  For PEERs, the toxics device ID used is the District’s PEER unit ID plus 5000.  A complete listing of facility permits and PEERs is provided in Appendix B.

[List each toxic emission source by type that has a District permit or PEER. Identify the permit units associated with each source type, list the substances emitted, and describe the method used to calculate emissions.

Add additional source groups as needed and delete any non-applicable sources from the examples below. Maintain the same outline format.]

[The District has provided a suggested discussion and sample calculations for your permitted and PEER equipment. Included are default methods for quantifying toxic emissions from these devices. You may propose alternate quantification methods to the District for review and approval. Please review and modify this plan as appropriate for your operation.]

## EXAMPLE: External Combustion (Boilers, Dehydrators, Heaters, Dryers), Natural Gas and/or LPG (Propane)

### Applicability

[List the toxics device ID(s) this emission source applies to. Refer to Table 1 for the device numbers.]

This section applies to toxics device(s) X, X, X and 500X.

### Process Description

[Provide a process description of the above emission source per unit or source group, as applicable.]

Example: The facility operates two natural gas-fired boilers, one LPG-fired boiler and one natural gas-fired dryer. The boilers are used to provide steam to the various processes at the facility. The dryer is used to dry the starch as part of the starch production process.

### Listed Substances

[Below are potential toxic emissions profiles. Delete profiles not used for this source type.]

Listed substances expected to be emitted are from the following District toxic emissions profiles:

- Profile #3 for natural gas external combustion boilers with heat input <10 MMbtu/hr

- Profile #6 for natural gas external combustion boilers with heat input from 10-100 MMBtu/hr

- Profile #8 for natural gas external combustion boilers with heat input >100 MMbtu/hr

- Profile #102 for LPG external combustion boilers with heat input <10 MMbtu/hr

- Profile #103 for LPG external combustion boilers with heat input from 10-100 MMBtu/hr

- Profile #104 for LPG external combustion boilers with heat input >100 MMbtu/hr

[If the source has SCR, add the following, otherwise delete.]

- Ammonia (CAS# 7664417) from the Selective Catalytic Reduction system

Further information is provided in Appendix C.

### Calculation Method

[Update units for the equation below as needed.]

San Joaquin Valley APCD approved emission factors will be used to quantify emissions of listed substances. Emissions of listed substances from the boilers will be quantified using the following equation:

ELS = EFLS x PR

where,

ELS 	= 	listed substance emissions in lb/yr or lb/hr

EFLS	= 	listed substance emission factor in lb/mmscf natural gas or 1,000 gallons LPG fuel

PR 	= 	process rate in mmscf natural gas or 1,000 gallons LPG fuel consumed per year or hour

[If the source has SCR, add the following, otherwise delete.]

Ammonia emissions shall be calculated in accordance with the requirements of the annual emissions inventory.

## EXAMPLE: Turbine IC Engines, Natural Gas and/or LPG (Propane)

### Applicability

[List the toxics device ID(s) this emission source applies to. Refer to Table 1 for the device numbers.]

This section applies to toxics device(s) X and X.

### Process Description

[Provide a process description of the above emission source per unit or source group, as applicable.]

Example: The facility operates two natural gas-fired turbines.  The turbines power electrical generators.

### Listed Substances

[Below are potential toxic emissions profiles. Delete profiles not used for this source type.]

Listed substances expected to be emitted are from the following District toxic emissions profile:

- Profile #162 natural gas-fired turbine without a catalyst

- Profile #163 natural gas-fired turbine with a catalyst

- Profile #108 LPG-fired turbine without a catalyst

- Profile #158 LPG-fired turbine with a catalyst

[If the turbine has SCR, add the following, otherwise delete.]

- Ammonia (CAS# 7664417) from the Selective Catalytic Reduction system

Further information is provided in Appendix C.

### Calculation Method

[Update units for the equation below as needed.]

San Joaquin Valley APCD approved emission factors will be used to quantify emissions of listed substances. Emissions of listed substances from the engines will be quantified using the following equation:

ELS = EFLS x PR

where,

ELS 	= 	listed substance emissions in lb/yr or lb/hr

EFLS	= 	listed substance emission factor in lb/mmscf natural gas or 1,000 gallons LPG fuel

PR 	= 	process rate in mmscf natural gas or 1,000 gallons LPG fuel consumed per year or hour

[If the turbine has SCR, add the following, otherwise delete.]

Ammonia emissions shall be calculated in accordance with the requirements of the annual emissions inventory.

## EXAMPLE: Reciprocating IC Engines, Diesel

### Applicability

[List the toxics device ID(s) this emission source applies to. Refer to Table 1 for the device numbers.]

This section applies to toxics device X.

### Process Description

[Provide a process description of the above emission source per unit or source group, as applicable.]

Example: The facility operates one diesel fired emergency standby engine.  The engine powers a generator which provides electrical power in case of an outage.

### Listed Substances

[List all toxic profiles associated with this emission source.]

Listed substances expected to be emitted are from the following District toxic emissions profile:

- Profile #136 diesel engine particulate matter

### Calculation Method

A San Joaquin Valley APCD PM10 speciation profile will be used to quantify emissions of listed substances. The PM10 emission factor used for reporting of the annual emissions inventory for the specified device will be used to estimate PM10 emissions.  The PM10 emissions will then be speciated into associated listed substances as follows:

ELS = EFPM10 x PR x SFLS

Where,

ELS 	= listed substance emissions in lb/yr or lb/hr

EFPM10 	= PM10 emission factor in lb/unit process

PR 	= process rate per year or hour

SFLS	= listed substance PM10 speciation factor (lb LS/lb PM10)

The PM10 emission factor will be from the engine’s certified value as recorded in the SJVAPCD permit to operate.

To comply with SCC unit reporting, submit the amount of fuel used in

1,000 gallons.

## EXAMPLE: Reciprocating IC Engines, Natural Gas and/or LPG (Propane)

### Applicability

[List the toxics device ID(s) this emission source applies to. Refer to Table 1 for the device numbers.]

This section applies to toxics device(s) X, X, and X.

### Process Description

[Provide a process description of the above emission source per unit or source group, as applicable.]

Example: The facility operates three natural gas-fired engines.  The engines power water pumps.

### Listed Substances

[Below are potential toxic emissions profiles. Delete profiles not used for this source type.]

Listed substances expected to be emitted are from the following District toxic emissions profile:

- Profile #161 for 4-stroke rich burn natural gas-fired engines

- Profile #160 for 4-stroke lean burn natural gas-fired engines

- Profile #159 for 2-stroke lean burn natural gas-fired engines

- Profile #156 for 4-stroke rich burn LPG-fired engines

- Profile #137 for 4-stroke lean burn LPG -fired engines

- Profile #154 for 2-stroke lean burn LPG -fired engines

[If the source has SCR, add the following, otherwise delete.]

- Ammonia (CAS# 7664417) from the Selective Catalytic Reduction system

Further information is provided in Appendix C.

### Calculation Method

[Update units for the equation below as needed.]

San Joaquin Valley APCD approved emission factors will be used to quantify emissions of listed substances. Emissions of listed substances from the engines will be quantified using the following equation:

ELS = EFLS-UNC x CF x PR

where,

ELS 		= 	listed substance emissions in lb/yr or lb/hr

EFLS-UNC	= 	listed substance uncontrolled emission factor in lb/unit 
	fuel consumed

CF		=	control factor

PR 		= 	process rate in mmscf natural gas or 1,000 gallons 
	LPG consumed per year or hour

The control factor will be calculated as follows:

CF	=	(VOCUNC - VOCCON) / VOCUNC

where,

VOCUNC	=	uncontrolled VOC emissions rate from the speciation profile selected

VOCCON	=	controlled VOC emission rate of the equipment from the District Permit or source test (if applicable)

[If the source has SCR, add the following, otherwise delete.]

Ammonia emissions shall be calculated in accordance with the requirements of the annual emissions inventory.

## EXAMPLE: Permitted PM10 Sources: Source-Specific Heading [Change the heading title to appropriately describe the source.]

### Applicability

[List the toxics device ID(s) this emission source applies to. Refer to Table 1 for the device numbers.]

This section applies to toxics device(s) X.

### Process Description

[Provide a process description of the above emission source per unit or source group, as applicable.]

### Listed Substances

[List all toxic profiles or toxic substances associated with this emission source.]

Example: Listed substances are components of XXX emitted during XXX operations.  Listed substances expected to be emitted are from the following District toxic emissions profile:

- Profile #XXX for XXX.

Further information is provided in Appendix C.

OR

[Examine Safety Data Sheets and determine if any listed substances are present. Provide name and CAS# in the table below for each listed substance present.]

Example: Listed substances are components of the particulate matter emitted during XXX.

Table 4.  Listed Substances Present in XXX

| Pollutant Name | CAS# |
| --- | --- |
| Example: Nickel | 7440020 |
| [Add more rows as necessary] |  |

### Calculation Method

[Provide proposed calculation method(s) for quantifying ACTUAL emissions.]

Example:

A San Joaquin Valley APCD speciation profile will be used to quantify emissions of listed substances. The PM10 emission factors (listed in the District’s permit to operate for the specified devices OR as established per AP-42 OR as established per project #XXXXXXX OR established by District-approved source tests OR Toxics Emissions Inventory Reports (TEIR) for the specified devices) [Update reference as needed] will be used to estimate PM10 emissions.  The PM10 emissions will then be speciated into associated listed substances as follows:

ELS = EFPM10 x PR x SFLS

where,

ELS 	= listed substance emissions in lb/yr or lb/hr

EFPM10 	= PM10 emission factor in lb/unit process

PR 	= process rate per year or hour

SFLS	= listed substance PM10 speciation factor (lb LS/lb PM10)

## EXAMPLE: Permitted VOC Sources: Source-Specific Heading [Change the heading title to appropriately describe the source.]

### Applicability

[List the toxics device ID(s) this emission source applies to. Refer to Table 1 for the device numbers.]

This section applies to toxics device(s) X and X.

### Process Description

[Provide a process description of the above emission source per unit or source group, as applicable.]

Example: The facility operates two XXXX. VOCs emitted are fugitive.

### Listed Substances

[List all toxic profiles or toxic substances associated with this emission source. The facility should identify and quantify any toxic emissions that may be contained in the VOC emissions released from the devices listed above.]

Example: Listed substances are components of fugitive VOCs emitted from XXXX.  Listed substances expected to be emitted are from the following District toxic emissions profile:

- Profile #XXX for XXXX

Further information is provided in Appendix C.

OR

Example: Listed substances from the XXX Safety Data Sheets are provided in the following table:

[Examine Safety Data Sheets and determine if any listed substances are present. Provide name and CAS# in the table below for each listed substance present.]

Table 5.  Listed Substances Present in XXX

| Pollutant Name | CAS# |
| --- | --- |
| Example: Ethyl Benzene | 100414 |
| [Add more rows as necessary] |  |

### Calculation Method

[Provide proposed calculation method(s) for quantifying ACTUAL emissions.]

Example:

San Joaquin Valley APCD approved emissions profile (speciation factors) will be used to quantify emissions of listed substances. The VOC emission factors (listed in the District’s permit to operate for the specified devices OR as established per AP-42 OR as established per project #XXXXXXX OR established by District-approved source tests OR Toxics Emissions Inventory Reports (TEIR) for the specified devices) [Update reference as needed] will be used to estimate VOCs.  The VOCs will then be speciated into associated listed substances as follows:

ELS = EFVOCx PR x EFLS

where,

ELS 	= listed substance emissions in lb/yr or lb/hr

EFVOC 	= VOC emission factor in lb/unit process

PR 	= process rate per year or hour

EFLS	= listed substance emission factor (lb LS/lb VOC)

## EXAMPLE: Other Permitted Sources [Change the heading title to appropriately describe the source.]

[Use this when we do not have a toxics profile for the source. The facility will determine substances and quantification methods.]

### Applicability

[List the toxics device ID(s) this emission source applies to.]

This section applies to toxics device(s) XX and XX.

### Process Description

[Provide a process description of the above emission source per unit or source group, as applicable.]

Example: The facility operates XXXXX.

### Listed Substances

[List all toxic substances associated with the emission source.]

### Calculation Method

[Provide proposed calculation method(s) for quantifying ACTUAL emissions.]

# Other Permit-Exempt Equipment

[List other permit-exempt equipment in this section, and describe the method by which toxic emissions will be calculated.  Delete this section if it does not apply. All PEERs are already accounted for in the previous section. Refer to Appendix A and Appendix C of the CARB Emission Inventory Criteria and Guidelines for details regarding the potential sources to include in this section. Please contact the District Hot Spots Team at (559) 230-6000 or at hotspots@valleyair.org with any questions you may have.]

The toxics device IDs used for these sources will start at 6001 and will continue sequentially (6001, 6002, 6003, etc.). Number each device separately.]

## EXAMPLE: [Other Permit-Exempt Equipment Type]

### Applicability

[List the toxics device ID(s) this emission source applies to.]

### Process Description

[Provide a process description of the above emission source per unit or source group, as applicable.]

### Listed Substances

[List all toxic profiles or toxic substances associated with this emission source.]

### Calculation Method

[Provide proposed calculation method(s) for quantifying ACTUAL emissions.]

# Other Permit-Exempt Fugitive Emissions

[List sources of fugitive emissions in this section and describe the method by which their toxic emissions will be calculated.  Fugitive emissions are those emissions which do not pass through a stack, chimney, vent or other functionally equivalent opening. Sources may include solvents, miscellaneous chemicals, leaks, etc. Fugitive emissions shall be reported with the permitted unit as applicable, otherwise fugitive emissions will be reported in this section. Delete this section if it does not apply. Refer to Appendix A and Appendix C of the CARB Emission Inventory Criteria and Guidelines for details regarding the potential sources to include in this section. Please contact the District Hot Spots Team at (559) 230-6000 or at hotspots@valleyair.org with any questions you may have.]

The toxics device ID used for these fugitive sources will start at 7001 and will continue sequentially (7001, 7002, 7003, etc.). Number each device separately.]

Safety Data Sheets (SDS) or other manufacturer's data will be used to determine the concentration of toxics in these substances. The toxics will be quantified based on engineering calculations, their physical properties, methods of application and amount of product used during the year.

## EXAMPLE: Chlorinated Water Processes

### Applicability

[List the toxics device ID(s) this emission source applies to.]

This section applies to toxics device(s) 700X and 700X.

### Process Description

[Provide a process description of the above emission source per unit or source group, as applicable.]

Example: Chlorine is added to both the plant’s cooling tower and decarbonators.  According to Appendix E-II-8 of the Technical Guidance Document for the Air Toxics “Hot Spots” Information and Assessment Act or 1987, the only hazardous air pollutant emissions that must be quantified from chlorinate water processes is chloroform.

### Listed Substances

[List all toxic substances emitted by the emission source.]

The listed substance expected to be emitted is chloroform (CAS# 67663).

### Calculation Method

[Provide proposed calculation method(s) for quantifying ACTUAL emissions.]

Example:

Emissions of chloroform from the chlorinated water processes will be calculated using the emission factor from ARB’s “Technical Guidance Document for the Emission Inventory Criteria and Guidelines Regulation for AB 2588, Attachment E-II-8” as follows:

E = U x EF

Where:

E	= chloroform emissions in lb/yr or lb/hr

U	= chlorine used in lb/yr or lb/hr

EF 	= chloroform emission factor (0.0034 lbs chloroform per lb chlorine)

## EXAMPLE: [Other Permit-Exempt Fugitive Emissions]

[Add additional sources of fugitive emissions as necessary in subsequent subsections 5.3, 5.4, etc.]

### Applicability

[List the toxics device ID(s) this emission source applies to.]

### Process Description

[Provide a process description of the above emission source per unit or source group, as applicable.]

### Listed Substances

[List all toxic profiles or toxic substances associated with this emission source.]

### Calculation Method

[Provide proposed calculation method(s) for quantifying ACTUAL emissions.]

# Source Testing

## No Source Testing

[If the available toxic emission profiles are a substantially equivalent characterization of facility-specific emissions, the facility may not need to propose new source testing; therefore, the following can be stated:]

Example: No new source testing is being proposed.

[Delete this section if 6.2: Proposed Source Testing is selected.]

## Proposed Source Testing

[Delete this section if 6.1: No Source Testing is selected.]

[Per Section IX of the Emission Inventory Criteria and Guidelines, source testing options include some of the following:

- Complete source testing in accordance with ARB-adopted source test methods

- Propose sampling and analysis methods that are substantially equivalent to ARB-adopted source test methods

- Use existing source test data from the facility

          - Use only if all conditions affecting emissions of listed substances are substantially the same

          - Existing source test methods must be equivalent to ARB-adopted test methods, as approved by the District

- Complete pooled source testing

          - A group of related facilities may perform a limited number of representative source tests to apply to their respective facilities

          - Utilize pooled source testing only if there is sufficient similarity in all emissions parameters between the facility tested and the facility applied to

- Propose an alternative method to quantify emissions that provides the best technologically feasible characterization of those facility emissions

          - The alternative method must result in a characterization that is as accurate as that achieved by the ARB-adopted source test method

          - Utilize this alternative  if physical circumstances at the facility do not allow for the ARB-adopted source test method

- Utilize ARB-approved emission factors from the California Air Toxics Emission Factors (CATEF) database, (subject to additional conditions)

          - https://www.arb.ca.gov/ei/catef/catef.htm]

If source testing is required or proposed, please list the emitting process, device, or facility activity for which source testing will be applied to. Refer to Section IX and Appendix D of the CARB Emission Inventory Criteria and Guidelines (www.arb.ca.gov/ab2588/2588guid.htm) for more details regarding source testing. The methods used to conduct source tests must be approved in advance by the District. If source testing is proposed, please attach the Source Testing Protocol, set forth in Section IX.E., to the Toxics Emission Inventory Plan.]

Example: Source testing is proposed for toxics device X, as detailed in the attached Source Testing Protocol.

Facility Process Flow Diagram(s)

[Each inventory plan shall include a flow diagram consisting of a comprehensive schematic drawing of the process flows that affect the nature or quantity of emissions of listed substances. If necessary, a reference document shall be attached to the drawing to include any information needed to fulfill the flow diagram requirements that cannot be included in the drawing. Flow diagram requirements are listed under Section VIB of the CARB Emission Inventory Criteria & Guidelines www.arb.ca.gov/ab2588/2588guid.htm. For information regarding trade secrets, please refer to Section VIC of the CARB Emission Inventory Criteria and Guidelines or the CA Health & Safety Code § 44346.]

EXAMPLE

Permit and PEER Unit Listing

| Permit ID | Toxics Device ID | Permit Status | Permit Description |
| --- | --- | --- | --- |

Toxic Emissions Profiles

| - Toxic Profile # | - Description |
| --- | --- |
| - 3 | - NG < 10 MMBTU/Hr External Combustion |
| - 6 | - NG 10-100 MMBTU/Hr External Combustion |
| - 8 | - NG >100 MMBTU/Hr External Combustion |
| - 102 | - LPG External Combustion- <10 MMBtu/hr |
| - 103 | - LPG External Combustion- 10-100 MMBtu/hr |
| - 104 | - LPG External Combustion->100 MMBtu/hr |
| - 108 | - LPG Internal Combustion - Turbine w/o Catalyst |
| - 136 | - Diesel Engine Particulate Matter |
| - 137 | - LPG-Fired Internal Combustion 4SLB Engine No Cont |
| - 154 | - LPG-Fired Internal Combustion 2SLB Engine No Cont |
| - 156 | - LPG-Fired Internal Combustion 4SRB Engine No Cont |
| - 158 | - LPG Internal Combustion - Turbine w/ Catalyst |
| - 159 | - NG Internal Combustion 2SLB Engine No Controls |
| - 160 | - NG Internal Combustion 4SLB Engine No Controls |
| - 161 | - NG Internal Combustion 4SRB Engine No Controls |
| - 162 | - NG Internal Combustion - Turbine w/o Catalyst |
| - 163 | - NG Internal Combustion - Turbine w/ Catalyst |

| District Toxic Profile ID | 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | NG < 10 MMBTU/Hr External Combustion |  |  |  |  |
| Source | The emission factors are from the table, "Natural Gas Fired External Combustion Equipment" in the May 2001 update of VCAPCD AB 2588 Combustion Emission Factors. |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| Acetaldehyde |  | 4.30E-03 | lb/mmscf | 75070 |
| Acrolein |  | 2.70E-03 | lb/mmscf | 107028 |
| Benzene |  | 8.00E-03 | lb/mmscf | 71432 |
| Ethyl benzene |  | 9.50E-03 | lb/mmscf | 100414 |
| Formaldehyde |  | 1.70E-02 | lb/mmscf | 50000 |
| Hexane |  | 6.30E-03 | lb/mmscf | 110543 |
| Naphthalene |  | 3.00E-04 | lb/mmscf | 91203 |
| PAHs, total, w/o individ. components reported |  | 1.00E-04 | lb/mmscf | 1151 |
| Propylene |  | 7.31E-01 | lb/mmscf | 115071 |
| Toluene |  | 3.66E-02 | lb/mmscf | 108883 |
| Xylenes (mixed) |  | 2.72E-02 | lb/mmscf | 1330207 |

| District Toxic Profile ID | 6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | NG 10-100 MMBTU/Hr External Combustion |  |  |  |  |
| Source | The emission factors are from the table, "Natural Gas Fired External Combustion Equipment" in the May 2001 update of VCAPCD AB 2588 Combustion Emission Factors. |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| Acetaldehyde |  | 3.10E-03 | lb/mmscf | 75070 |
| Acrolein |  | 2.70E-03 | lb/mmscf | 107028 |
| Benzene |  | 5.80E-03 | lb/mmscf | 71432 |
| Ethyl benzene |  | 6.90E-03 | lb/mmscf | 100414 |
| Formaldehyde |  | 1.23E-02 | lb/mmscf | 50000 |
| Hexane |  | 4.60E-03 | lb/mmscf | 110543 |
| Naphthalene |  | 3.00E-04 | lb/mmscf | 91203 |
| PAHs, total, w/o individ. components reported |  | 1.00E-04 | lb/mmscf | 1151 |
| Propylene |  | 5.30E-01 | lb/mmscf | 115071 |
| Toluene |  | 2.65E-02 | lb/mmscf | 108883 |
| Xylenes (mixed) |  | 1.97E-02 | lb/mmscf | 1330207 |

| District Toxic Profile ID | 8 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | NG >100 MMBTU/Hr External Combustion |  |  |  |  |
| Source | The emission factors are from the table, "Natural Gas Fired External Combustion Equipment" in the May 2001 update of VCAPCD AB 2588 Combustion Emission Factors. |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| Acetaldehyde |  | 9.00E-04 | lb/mmscf | 75070 |
| Acrolein |  | 8.00E-04 | lb/mmscf | 107028 |
| Benzene |  | 1.70E-03 | lb/mmscf | 71432 |
| Ethyl benzene |  | 2.00E-03 | lb/mmscf | 100414 |
| Formaldehyde |  | 3.60E-03 | lb/mmscf | 50000 |
| Hexane |  | 1.30E-03 | lb/mmscf | 110543 |
| Naphthalene |  | 3.00E-04 | lb/mmscf | 91203 |
| PAHs, total, w/o individ. components reported |  | 1.00E-04 | lb/mmscf | 1151 |
| Propylene |  | 1.55E-02 | lb/mmscf | 115071 |
| Toluene |  | 7.80E-03 | lb/mmscf | 108883 |
| Xylenes (mixed) |  | 5.80E-03 | lb/mmscf | 1330207 |

| District Toxic Profile ID | 102 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | LPG External Combustion- <10 MMBtu/hr |  |  |  |  |
| Source | The emission factors are from the table, "Natural Gas Fired External Combustion Equipment" in the May 2001 report, VCAPCD AB 2588 Combustion Emission Factors and a conversion from NG to LPG using District factors. |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| Acetaldehyde |  | 4.04E-04 | lb/1000 gal LPG | 75070 |
| Acrolein |  | 2.54E-04 | lb/1000 gal LPG | 107028 |
| Benzene |  | 7.52E-04 | lb/1000 gal LPG | 71432 |
| Ethyl benzene |  | 8.93E-04 | lb/1000 gal LPG | 100414 |
| Formaldehyde |  | 1.60E-03 | lb/1000 gal LPG | 50000 |
| Hexane |  | 5.92E-04 | lb/1000 gal LPG | 110543 |
| Naphthalene |  | 2.82E-05 | lb/1000 gal LPG | 91203 |
| PAHs, total, w/o individ. components reported |  | 9.40E-06 | lb/1000 gal LPG | 1151 |
| Propylene |  | 6.87E-02 | lb/1000 gal LPG | 115071 |
| Toluene |  | 3.44E-03 | lb/1000 gal LPG | 108883 |
| Xylenes (mixed) |  | 2.56E-03 | lb/1000 gal LPG | 1330207 |

| District Toxic Profile ID | 103 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | LPG External Combustion- 10-100 MMBtu/hr |  |  |  |  |
| Source | The emission factors are from the table, "Natural Gas Fired External Combustion Equipment" in the May 2001 update of VCAPCD AB 2588 Combustion Emission Factors and a conversion from NG to LPG using District factors. |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| Acetaldehyde |  | 2.91E-04 | lb/1000 gal LPG | 75070 |
| Acrolein |  | 2.54E-04 | lb/1000 gal LPG | 107028 |
| Benzene |  | 5.45E-04 | lb/1000 gal LPG | 71432 |
| Ethyl benzene |  | 6.49E-04 | lb/1000 gal LPG | 100414 |
| Formaldehyde |  | 1.16E-03 | lb/1000 gal LPG | 50000 |
| Hexane |  | 4.32E-04 | lb/1000 gal LPG | 110543 |
| Naphthalene |  | 2.82E-05 | lb/1000 gal LPG | 91203 |
| PAHs, total, w/o individ. components reported |  | 9.40E-06 | lb/1000 gal LPG | 1151 |
| Propylene |  | 4.98E-02 | lb/1000 gal LPG | 115071 |
| Toluene |  | 2.49E-03 | lb/1000 gal LPG | 108883 |
| Xylenes (mixed) |  | 1.85E-03 | lb/1000 gal LPG | 1330207 |

| District Toxic Profile ID | 104 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | LPG External Combustion->100 MMBtu/hr |  |  |  |  |
| Source | The emission factors are from the table, "Natural Gas Fired External Combustion Equipment" in the May 2001 update of VCAPCD AB 2588 Combustion Emission Factors and a conversion from NG to LPG using District factors. |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| Acetaldehyde |  | 8.46E-05 | lb/1000 gal LPG | 75070 |
| Acrolein |  | 7.52E-05 | lb/1000 gal LPG | 107028 |
| Benzene |  | 1.60E-04 | lb/1000 gal LPG | 71432 |
| Ethyl benzene |  | 1.88E-04 | lb/1000 gal LPG | 100414 |
| Formaldehyde |  | 3.38E-04 | lb/1000 gal LPG | 50000 |
| Hexane |  | 1.22E-04 | lb/1000 gal LPG | 110543 |
| Naphthalene |  | 2.82E-05 | lb/1000 gal LPG | 91203 |
| PAHs, total, w/o individ. components reported |  | 9.40E-06 | lb/1000 gal LPG | 1151 |
| Propylene |  | 1.46E-03 | lb/1000 gal LPG | 115071 |
| Toluene |  | 7.33E-04 | lb/1000 gal LPG | 108883 |
| Xylenes (mixed) |  | 5.45E-04 | lb/1000 gal LPG | 1330207 |

| District Toxic Profile ID | 108 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | LPG Internal Combustion - Turbine w/o Catalyst |  |  |  |  |
| Source | LPG-fired turbine toxic emission are not available, so natural gas-fired turbine emission factors are used as a surrogate.The emission factors were based on AP 42, Fifth Edition, Volume I, Chapter 3: Stationary Internal Combustion Sources, Section 3: Stationary Gas Turbines, Table 3.1-3. Assumes 94,000 Btu/gal LPG |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| 1,3-Butadiene |  | 4.04E-05 | lb/1000 gal LPG | 106990 |
| Acetaldehyde |  | 3.76E-03 | lb/1000 gal LPG | 75070 |
| Acrolein |  | 6.02E-04 | lb/1000 gal LPG | 107028 |
| Benzene |  | 1.13E-03 | lb/1000 gal LPG | 71432 |
| Ethyl benzene |  | 3.01E-03 | lb/1000 gal LPG | 100414 |
| Formaldehyde |  | 6.67E-02 | lb/1000 gal LPG | 50000 |
| Naphthalene |  | 1.22E-04 | lb/1000 gal LPG | 91203 |
| PAHs, total, with individ. components also reported |  | 2.07E-04 | lb/1000 gal LPG | 1150 |
| Propylene oxide |  | 2.73E-03 | lb/1000 gal LPG | 75569 |
| Toluene |  | 1.22E-02 | lb/1000 gal LPG | 108883 |
| Xylenes (mixed) |  | 6.02E-03 | lb/1000 gal LPG | 1330207 |

| District Toxic Profile ID | 136 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Diesel Engine Particulate Matter |  |  |  |  |
| Source | Assumes all PM10 is DPM |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| Diesel engine exhaust, particulate matter |  | 1.00E+00 | lb/lb PM10 | 9901 |

| District Toxic Profile ID | 137 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | LPG-Fired Internal Combustion 4SLB Engine No Cont |  |  |  |  |
| Source | The emission factors are derived from July 2000 AP 42, Fifth Edition, Volume I, Chapter 3: Stationary Internal Combustion Sources, Section 2: Natural Gas-Fired Reciprocating Engine and conversion from NG to LPG using District conversion factors.. Use spreadsheet to determine VOC control or to use different HHV |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| 1,1,2,2-Tetrachloroethane |  | 3.76E-03 | lb/1,000 gallons | 79345 |
| 1,1,2-Trichloroethane |  | 2.99E-03 | lb/1,000 gallons | 79005 |
| 1,1-Dichloroethane |  | 2.22E-03 | lb/1,000 gallons | 75343 |
| 1,2,4-Trimethylbenzene |  | 1.34E-03 | lb/1,000 gallons | 95636 |
| 1,3-Butadiene |  | 2.51E-02 | lb/1,000 gallons | 106990 |
| 2,2,4-Trimethylpentane |  | 2.35E-02 | lb/1,000 gallons | 540841 |
| 2-Methyl naphthalene |  | 3.12E-03 | lb/1,000 gallons | 91576 |
| Acenaphthene |  | 1.18E-04 | lb/1,000 gallons | 83329 |
| Acenaphthylene |  | 5.20E-04 | lb/1,000 gallons | 208968 |
| Acetaldehyde |  | 7.86E-01 | lb/1,000 gallons | 75070 |
| Acrolein |  | 4.83E-01 | lb/1,000 gallons | 107028 |
| Benzene |  | 4.14E-02 | lb/1,000 gallons | 71432 |
| Benzo[b]fluoranthene |  | 1.56E-05 | lb/1,000 gallons | 205992 |
| Benzo[e]pyrene |  | 3.90E-05 | lb/1,000 gallons | 192972 |
| Benzo[g,h,i]perylene |  | 3.89E-05 | lb/1,000 gallons | 191242 |
| Biphenyl |  | 1.99E-02 | lb/1,000 gallons | 92524 |
| Carbon tetrachloride |  | 3.45E-03 | lb/1,000 gallons | 56235 |
| Chlorobenzene |  | 2.86E-03 | lb/1,000 gallons | 108907 |
| Chloroform |  | 2.68E-03 | lb/1,000 gallons | 67663 |
| Chrysene |  | 6.51E-05 | lb/1,000 gallons | 218019 |
| Ethyl benzene |  | 3.73E-03 | lb/1,000 gallons | 100414 |
| Ethylene dibromide {EDB} |  | 4.16E-03 | lb/1,000 gallons | 106934 |
| Fluoranthene |  | 1.04E-04 | lb/1,000 gallons | 206440 |
| Fluorene |  | 5.33E-04 | lb/1,000 gallons | 86737 |
| Formaldehyde |  | 4.96E+00 | lb/1,000 gallons | 50000 |
| Hexane |  | 1.04E-01 | lb/1,000 gallons | 110543 |
| Methanol |  | 2.35E-01 | lb/1,000 gallons | 67561 |
| Methylene chloride {Dichloromethane} |  | 1.88E-03 | lb/1,000 gallons | 75092 |
| Naphthalene |  | 6.99E-03 | lb/1,000 gallons | 91203 |
| PAHs, total, w/o individ. components reported |  | 2.53E-03 | lb/1,000 gallons | 1151 |
| Phenanthrene |  | 9.78E-04 | lb/1,000 gallons | 85018 |
| Phenol |  | 2.26E-03 | lb/1,000 gallons | 108952 |
| Pyrene |  | 1.28E-04 | lb/1,000 gallons | 129000 |
| Styrene |  | 2.22E-03 | lb/1,000 gallons | 100425 |
| Toluene |  | 3.84E-02 | lb/1,000 gallons | 108883 |
| Vinyl chloride |  | 1.40E-03 | lb/1,000 gallons | 75014 |
| Xylenes (mixed) |  | 1.73E-02 | lb/1,000 gallons | 1330207 |

| District Toxic Profile ID | 154 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | LPG-Fired Internal Combustion 2SLB Engine No Cont |  |  |  |  |
| Source | The emission factors are derived from July 2000 AP 42, Fifth Edition, Volume I, Chapter 3: Stationary Internal Combustion Sources, Section 2: Natural Gas-Fired Reciprocating Engine and conversion from NG to LPG using District conversion factors.. Use spreadsheet to determine VOC control or to use different HHV |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| 1,1,2,2-Tetrachloroethane |  | 6.23E-03 | lbs/1,000 gals | 79345 |
| 1,1,2-Trichloroethane |  | 4.95E-03 | lbs/1,000 gals | 79005 |
| 1,1-Dichloroethane |  | 3.68E-03 | lbs/1,000 gals | 75343 |
| 1,2,4-Trimethylbenzene |  | 9.21E-04 | lbs/1,000 gals | 95636 |
| 1,3-Butadiene |  | 7.71E-02 | lbs/1,000 gals | 106990 |
| 2,2,4-Trimethylpentane |  | 7.95E-02 | lbs/1,000 gals | 540841 |
| 2-Methyl naphthalene |  | 2.01E-03 | lbs/1,000 gals | 91576 |
| Acenaphthene |  | 1.25E-04 | lbs/1,000 gals | 83329 |
| Acenaphthylene |  | 2.98E-04 | lbs/1,000 gals | 208968 |
| Acetaldehyde |  | 7.29E-01 | lbs/1,000 gals | 75070 |
| Acrolein |  | 7.31E-01 | lbs/1,000 gals | 107028 |
| Anthracene |  | 6.75E-05 | lbs/1,000 gals | 120127 |
| Benz[a]anthracene |  | 3.16E-05 | lbs/1,000 gals | 56553 |
| Benzene |  | 1.82E-01 | lbs/1,000 gals | 71432 |
| Benzo[a]pyrene |  | 2.20E-06 | lbs/1,000 gals | 50328 |
| Benzo[b]fluoranthene |  | 8.00E-07 | lbs/1,000 gals | 205992 |
| Benzo[e]pyrene |  | 2.20E-06 | lbs/1,000 gals | 192972 |
| Benzo[g,h,i]perylene |  | 2.33E-06 | lbs/1,000 gals | 191242 |
| Benzo[k]fluoranthene |  | 4.00E-07 | lbs/1,000 gals | 207089 |
| Biphenyl |  | 3.71E-04 | lbs/1,000 gals | 92524 |
| Carbon tetrachloride |  | 5.71E-03 | lbs/1,000 gals | 56235 |
| Chlorobenzene |  | 4.17E-03 | lbs/1,000 gals | 108907 |
| Chloroform |  | 4.43E-03 | lbs/1,000 gals | 67663 |
| Chrysene |  | 6.32E-05 | lbs/1,000 gals | 218019 |
| Cyclohexane |  | 2.90E-02 | lbs/1,000 gals | 110827 |
| Ethyl benzene |  | 1.02E-02 | lbs/1,000 gals | 100414 |
| Ethylene dibromide {EDB} |  | 6.90E-03 | lbs/1,000 gals | 106934 |
| Fluoranthene |  | 3.39E-05 | lbs/1,000 gals | 206440 |
| Fluorene |  | 1.59E-04 | lbs/1,000 gals | 86737 |
| Formaldehyde |  | 5.19E+00 | lbs/1,000 gals | 50000 |
| Hexane |  | 4.18E-02 | lbs/1,000 gals | 110543 |
| Indeno[1,2,3-cd]pyrene |  | 9.33E-07 | lbs/1,000 gals | 193395 |
| Methanol |  | 2.33E-01 | lbs/1,000 gals | 67561 |
| Methylene chloride {Dichloromethane} |  | 1.38E-02 | lbs/1,000 gals | 75092 |
| Naphthalene |  | 9.05E-03 | lbs/1,000 gals | 91203 |
| PAHs, total, w/o individ. components reported |  | 1.23E-02 | lbs/1,000 gals | 1151 |
| Perylene |  | 4.67E-07 | lbs/1,000 gals | 198550 |
| Phenanthrene |  | 3.32E-04 | lbs/1,000 gals | 85018 |
| Phenol |  | 3.96E-03 | lbs/1,000 gals | 108952 |
| Pyrene |  | 5.49E-05 | lbs/1,000 gals | 129000 |
| Styrene |  | 5.15E-03 | lbs/1,000 gals | 100425 |
| Toluene |  | 9.05E-02 | lbs/1,000 gals | 108883 |
| Vinyl chloride |  | 2.32E-03 | lbs/1,000 gals | 75014 |
| Xylenes (mixed) |  | 2.52E-02 | lbs/1,000 gals | 1330207 |

| District Toxic Profile ID | 156 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | LPG-Fired Internal Combustion 4SRB Engine No Cont |  |  |  |  |
| Source | The emission factors are derived from July 2000 AP 42, Fifth Edition, Volume I, Chapter 3: Stationary Internal Combustion Sources, Section 2: Natural Gas-Fired Reciprocating Engine and conversion from NG to LPG using District conversion factors. Use spreadsheet to determine VOC control or to use different HHV. |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| 1,1,2,2-Tetrachloroethane |  | 2.38E-03 | lbs/1,000 gals | 79345 |
| 1,1,2-Trichloroethane |  | 1.44E-03 | lbs/1,000 gals | 79005 |
| 1,1-Dichloroethane |  | 1.06E-03 | lbs/1,000 gals | 75343 |
| 1,3-Butadiene |  | 6.23E-02 | lbs/1,000 gals | 106990 |
| Acetaldehyde |  | 2.62E-01 | lbs/1,000 gals | 75070 |
| Acrolein |  | 2.47E-01 | lbs/1,000 gals | 107028 |
| Benzene |  | 1.49E-01 | lbs/1,000 gals | 71432 |
| Carbon tetrachloride |  | 1.66E-03 | lbs/1,000 gals | 56235 |
| Chlorobenzene |  | 1.21E-03 | lbs/1,000 gals | 108907 |
| Chloroform |  | 1.29E-03 | lbs/1,000 gals | 67663 |
| Ethyl benzene |  | 2.33E-03 | lbs/1,000 gals | 100414 |
| Ethylene dibromide {EDB} |  | 2.00E-03 | lbs/1,000 gals | 106934 |
| Formaldehyde |  | 1.93E+00 | lbs/1,000 gals | 50000 |
| Methanol |  | 2.88E-01 | lbs/1,000 gals | 67561 |
| Methylene chloride {Dichloromethane} |  | 3.87E-03 | lbs/1,000 gals | 75092 |
| Naphthalene |  | 9.13E-03 | lbs/1,000 gals | 91203 |
| PAHs, total, w/o individ. components reported |  | 1.33E-02 | lbs/1,000 gals | 1151 |
| Styrene |  | 1.12E-03 | lbs/1,000 gals | 100425 |
| Toluene |  | 5.25E-02 | lbs/1,000 gals | 108883 |
| Vinyl chloride |  | 6.75E-04 | lbs/1,000 gals | 75014 |
| Xylenes (mixed) |  | 1.83E-02 | lbs/1,000 gals | 1330207 |

| District Toxic Profile ID | 158 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | LPG Internal Combustion - Turbine w/ Catalyst |  |  |  |  |
| Source | LPG-fired turbine toxic emission are not available, so natural gas-fired turbine emission factors are used as a surrogate.The emission factors were based on AP 42, Fifth Edition, Volume I, Chapter 3: Stationary Internal Combustion Sources, Section 3: Stationary Gas Turbines, Table 3.1-3. Assumes 94,000 Btu/gal LPG |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| 1,3-Butadiene |  | 4.04E-05 | lb/1,000 Gallons | 106990 |
| Acetaldehyde |  | 3.76E-03 | lb/1,000 Gallons | 75070 |
| Acrolein |  | 6.02E-04 | lb/1,000 Gallons | 107028 |
| Benzene |  | 8.55E-05 | lb/1,000 Gallons | 71432 |
| Ethyl benzene |  | 3.01E-03 | lb/1,000 Gallons | 100414 |
| Formaldehyde |  | 1.88E-03 | lb/1,000 Gallons | 50000 |
| Naphthalene |  | 1.22E-04 | lb/1,000 Gallons | 91203 |
| PAHs, total, with individ. components also reported |  | 2.07E-04 | lb/1,000 Gallons | 1150 |
| Propylene oxide |  | 2.73E-03 | lb/1,000 Gallons | 75569 |
| Toluene |  | 1.22E-02 | lb/1,000 Gallons | 108883 |
| Xylenes (mixed) |  | 6.02E-03 | lb/1,000 Gallons | 1330207 |

| District Toxic Profile ID | 159 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | NG Internal Combustion 2SLB Engine No Controls |  |  |  |  |
| Source | The emission factors were based on AP 42, Fifth Edition, Volume I, Chapter 3: Stationary Internal Combustion Sources, Section 2: Natural Gas-Fired Reciprocating Engines, Table 3.2-1. Assumes 1,000 btu per scf natural gas. Use spreadsheet to determine VOC control or to use different HHV |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| 1,1,2,2-Tetrachloroethane |  | 6.63E-02 | lb/MMscf | 79345 |
| 1,1,2-Trichloroethane |  | 5.27E-02 | lb/MMscf | 79005 |
| 1,1-Dichloroethane |  | 3.91E-02 | lb/MMscf | 75343 |
| 1,2,4-Trimethylbenzene |  | 9.80E-03 | lb/MMscf | 95636 |
| 1,3-Butadiene |  | 8.20E-01 | lb/MMscf | 106990 |
| 2,2,4-Trimethylpentane |  | 8.46E-01 | lb/MMscf | 540841 |
| 2-Methyl naphthalene |  | 2.14E-02 | lb/MMscf | 91576 |
| Acenaphthene |  | 1.33E-03 | lb/MMscf | 83329 |
| Acenaphthylene |  | 3.17E-03 | lb/MMscf | 208968 |
| Acetaldehyde |  | 7.76E+00 | lb/MMscf | 75070 |
| Acrolein |  | 7.78E+00 | lb/MMscf | 107028 |
| Anthracene |  | 7.18E-04 | lb/MMscf | 120127 |
| Benz[a]anthracene |  | 3.36E-04 | lb/MMscf | 56553 |
| Benzene |  | 1.94E+00 | lb/MMscf | 71432 |
| Benzo[a]pyrene |  | 5.68E-06 | lb/MMscf | 50328 |
| Benzo[b]fluoranthene |  | 8.51E-06 | lb/MMscf | 205992 |
| Benzo[e]pyrene |  | 2.34E-05 | lb/MMscf | 192972 |
| Benzo[g,h,i]perylene |  | 2.48E-05 | lb/MMscf | 191242 |
| Benzo[k]fluoranthene |  | 4.26E-06 | lb/MMscf | 207089 |
| Biphenyl |  | 3.95E-03 | lb/MMscf | 92524 |
| Carbon tetrachloride |  | 6.07E-02 | lb/MMscf | 56235 |
| Chlorobenzene |  | 4.44E-02 | lb/MMscf | 108907 |
| Chloroform |  | 4.71E-02 | lb/MMscf | 67663 |
| Chrysene |  | 6.72E-04 | lb/MMscf | 218019 |
| Cyclohexane |  | 3.08E-01 | lb/MMscf | 110827 |
| Ethyl benzene |  | 1.08E-01 | lb/MMscf | 100414 |
| Ethylene dibromide {EDB} |  | 7.34E-02 | lb/MMscf | 106934 |
| Ethylene dichloride {EDC} |  | 4.22E-02 | lb/MMscf | 107062 |
| Fluoranthene |  | 3.61E-04 | lb/MMscf | 206440 |
| Fluorene |  | 1.69E-03 | lb/MMscf | 86737 |
| Formaldehyde |  | 5.52E+01 | lb/MMscf | 50000 |
| Hexane |  | 4.45E-01 | lb/MMscf | 110543 |
| Indeno[1,2,3-cd]pyrene |  | 9.93E-06 | lb/MMscf | 193395 |
| Methanol |  | 2.48E+00 | lb/MMscf | 67561 |
| Methylene chloride {Dichloromethane} |  | 1.47E-01 | lb/MMscf | 75092 |
| Naphthalene |  | 9.63E-02 | lb/MMscf | 91203 |
| PAHs, total, w/o individ. components reported |  | 3.47E-02 | lb/MMscf | 1151 |
| Perylene |  | 4.97E-06 | lb/MMscf | 198550 |
| Phenanthrene |  | 3.53E-03 | lb/MMscf | 85018 |
| Phenol |  | 4.21E-02 | lb/MMscf | 108952 |
| Pyrene |  | 5.84E-04 | lb/MMscf | 129000 |
| Styrene |  | 5.48E-02 | lb/MMscf | 100425 |
| Toluene |  | 9.63E-01 | lb/MMscf | 108883 |
| Vinyl chloride |  | 2.47E-02 | lb/MMscf | 75014 |
| Xylenes (mixed) |  | 2.68E-01 | lb/MMscf | 1330207 |

| District Toxic Profile ID | 160 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | NG Internal Combustion 4SLB Engine No Controls |  |  |  |  |
| Source | The emission factors were based on AP 42, Fifth Edition, Volume I, Chapter 3: Stationary Internal Combustion Sources, Section 2: Natural Gas-Fired Reciprocating Engines, Table 3.2-2. Assumes 1,000 Btu per scf natural gas. Use spreadsheet to determine VOC control or to use different HHV |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| 1,1,2,2-Tetrachloroethane |  | 4.00E-02 | lb/MMscf | 79345 |
| 1,1,2-Trichloroethane |  | 3.18E-02 | lb/MMscf | 79005 |
| 1,1-Dichloroethane |  | 2.36E-02 | lb/MMscf | 75343 |
| 1,2,4-Trimethylbenzene |  | 1.43E-02 | lb/MMscf | 95636 |
| 1,3-Butadiene |  | 2.67E-01 | lb/MMscf | 106990 |
| 2,2,4-Trimethylpentane |  | 2.50E-01 | lb/MMscf | 540841 |
| 2-Methyl naphthalene |  | 3.32E-02 | lb/MMscf | 91576 |
| Acenaphthene |  | 1.25E-03 | lb/MMscf | 83329 |
| Acenaphthylene |  | 5.53E-03 | lb/MMscf | 208968 |
| Acetaldehyde |  | 8.36E+00 | lb/MMscf | 75070 |
| Acrolein |  | 5.14E+00 | lb/MMscf | 107028 |
| Benzene |  | 4.40E-01 | lb/MMscf | 71432 |
| Benzo[b]fluoranthene |  | 1.66E-04 | lb/MMscf | 205992 |
| Benzo[e]pyrene |  | 4.15E-04 | lb/MMscf | 192972 |
| Benzo[g,h,i]perylene |  | 4.14E-04 | lb/MMscf | 191242 |
| Biphenyl |  | 2.12E-01 | lb/MMscf | 92524 |
| Carbon tetrachloride |  | 3.67E-02 | lb/MMscf | 56235 |
| Chlorobenzene |  | 3.04E-02 | lb/MMscf | 108907 |
| Chloroform |  | 2.85E-02 | lb/MMscf | 67663 |
| Chrysene |  | 6.93E-04 | lb/MMscf | 218019 |
| Ethyl benzene |  | 3.97E-02 | lb/MMscf | 100414 |
| Ethylene dibromide {EDB} |  | 4.43E-02 | lb/MMscf | 106934 |
| Ethylene dichloride {EDC} |  | 2.36E-02 | lb/MMscf | 107062 |
| Fluoranthene |  | 1.11E-03 | lb/MMscf | 206440 |
| Fluorene |  | 5.67E-03 | lb/MMscf | 86737 |
| Formaldehyde |  | 5.28E+01 | lb/MMscf | 50000 |
| Hexane |  | 1.11E+00 | lb/MMscf | 110543 |
| Methanol |  | 2.50E+00 | lb/MMscf | 67561 |
| Methylene chloride {Dichloromethane} |  | 2.00E-02 | lb/MMscf | 75092 |
| Naphthalene |  | 7.44E-02 | lb/MMscf | 91203 |
| PAHs, total, w/o individ. components reported |  | 7.75E-03 | lb/MMscf | 1151 |
| Phenanthrene |  | 1.04E-02 | lb/MMscf | 85018 |
| Phenol |  | 2.40E-02 | lb/MMscf | 108952 |
| Pyrene |  | 1.36E-03 | lb/MMscf | 129000 |
| Styrene |  | 2.36E-02 | lb/MMscf | 100425 |
| Toluene |  | 4.08E-01 | lb/MMscf | 108883 |
| Vinyl chloride |  | 1.49E-02 | lb/MMscf | 75014 |
| Xylenes (mixed) |  | 1.84E-01 | lb/MMscf | 1330207 |

| District Toxic Profile ID | 161 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | NG Internal Combustion 4SRB Engine No Controls |  |  |  |  |
| Source | The emission factors were based on AP 42, Fifth Edition, Volume I, Chapter 3: Stationary Internal Combustion Sources, Section 2: Natural Gas-Fired Reciprocating Engines, Table 3.2-3. Assumes 1,000 Btu's per scf natural gas. Use spreadsheet to determine VOC control or to use different HHV |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| 1,1,2,2-Tetrachloroethane |  | 2.53E-02 | lbs/MMscf | 79345 |
| 1,1,2-Trichloroethane |  | 1.53E-02 | lbs/MMscf | 79005 |
| 1,1-Dichloroethane |  | 1.13E-02 | lbs/MMscf | 75343 |
| 1,3-Butadiene |  | 6.63E-01 | lbs/MMscf | 106990 |
| Acetaldehyde |  | 2.79E+00 | lbs/MMscf | 75070 |
| Acrolein |  | 2.63E+00 | lbs/MMscf | 107028 |
| Benzene |  | 1.58E+00 | lbs/MMscf | 71432 |
| Carbon tetrachloride |  | 1.77E-02 | lbs/MMscf | 56235 |
| Chlorobenzene |  | 1.29E-02 | lbs/MMscf | 108907 |
| Chloroform |  | 1.37E-02 | lbs/MMscf | 67663 |
| Ethyl benzene |  | 2.48E-02 | lbs/MMscf | 100414 |
| Ethylene dibromide {EDB} |  | 2.13E-02 | lbs/MMscf | 106934 |
| Ethylene dichloride {EDC} |  | 1.13E-02 | lbs/MMscf | 107062 |
| Formaldehyde |  | 2.05E+01 | lbs/MMscf | 50000 |
| Methanol |  | 3.06E+00 | lbs/MMscf | 67561 |
| Methylene chloride {Dichloromethane} |  | 4.12E-02 | lbs/MMscf | 75092 |
| Naphthalene |  | 9.71E-02 | lbs/MMscf | 91203 |
| PAHs, total, w/o individ. components reported |  | 4.39E-02 | lbs/MMscf | 1151 |
| Styrene |  | 1.19E-02 | lbs/MMscf | 100425 |
| Toluene |  | 5.58E-01 | lbs/MMscf | 108883 |
| Vinyl chloride |  | 7.18E-03 | lbs/MMscf | 75014 |
| Xylenes (mixed) |  | 1.95E-01 | lbs/MMscf | 1330207 |

| District Toxic Profile ID | 162 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | NG Internal Combustion - Turbine w/o Catalyst |  |  |  |  |
| Source | The emission factors were based on AP 42, Fifth Edition, Volume I, Chapter 3: Stationary Internal Combustion Sources, Section 3: Stationary Gas Turbines, Table 3.1-3. Assumes 1,000 Btu's per scf natural gas. |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| 1,3-Butadiene |  | 4.30E-04 | lb/MMscf | 106990 |
| Acetaldehyde |  | 4.00E-02 | lb/MMscf | 75070 |
| Acrolein |  | 6.40E-03 | lb/MMscf | 107028 |
| Benzene |  | 1.20E-02 | lb/MMscf | 71432 |
| Ethyl benzene |  | 3.20E-02 | lb/MMscf | 100414 |
| Formaldehyde |  | 7.10E-01 | lb/MMscf | 50000 |
| Naphthalene |  | 1.30E-03 | lb/MMscf | 91203 |
| PAHs, total, with individ. components also reported |  | 2.20E-03 | lb/MMscf | 1150 |
| Propylene oxide |  | 2.90E-02 | lb/MMscf | 75569 |
| Toluene |  | 1.30E-01 | lb/MMscf | 108883 |
| Xylenes (mixed) |  | 6.40E-02 | lb/MMscf | 1330207 |

| District Toxic Profile ID | 163 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | NG Internal Combustion - Turbine w/ Catalyst |  |  |  |  |
| Source | The emission factors were based on AP 42, Fifth Edition, Volume I, Chapter 3: Stationary Internal Combustion Sources, Section 3: Stationary Gas Turbines, Table 3.1-3. Assumes 1,000 Btu's per scf natural gas. |  |  |  |  |
| Pollutant Name |  | Emission Factor | Emission Factor Units | CAS# |
| 1,3-Butadiene |  | 4.30E-04 | lbs/MMscf | 106990 |
| Acetaldehyde |  | 4.00E-02 | lbs/MMscf | 75070 |
| Acrolein |  | 6.40E-03 | lbs/MMscf | 107028 |
| Benzene |  | 9.10E-04 | lbs/MMscf | 71432 |
| Ethyl benzene |  | 3.20E-02 | lbs/MMscf | 100414 |
| Formaldehyde |  | 2.00E-02 | lbs/MMscf | 50000 |
| Naphthalene |  | 1.30E-03 | lbs/MMscf | 91203 |
| PAHs, total, with individ. components also reported |  | 2.20E-03 | lbs/MMscf | 1150 |
| Propylene oxide |  | 2.90E-02 | lbs/MMscf | 75569 |
| Toluene |  | 1.30E-01 | lbs/MMscf | 108883 |
| Xylenes (mixed) |  | 6.40E-02 | lbs/MMscf | 1330207 |

Safety Data Sheets

[Identify the toxic device(s) to which each SDS is associated with. Provide all SDSs as applicable.]

Stack Parameters

[If no exhaust stacks are present, include the following:]

Not applicable

[Otherwise, provide exhaust stack parameters in the table below:]

| Stack ID | Height (ft) | Diameter (ft) | Exhaust Temperature (F) | Gas Flow Rate (ACFM) | Gas Exit Velocity (ft/min) |
| --- | --- | --- | --- | --- | --- |

Facility Map with Receptor Locations

[Include a facility map that shows the site and structure(s).  The map must indicate the direction of north and indicate scale.  Annotated satellite images are acceptable as long as they are legible.  All sources of toxic emissions (devices) must be shown and referenced by the device numbers used in the Plan.  General locations of fugitive emissions must be noted.  The map must also identify the nearest potential receptors, including but not limited to the nearest hospital, school, daycare center, worksite, and residence.  The distance to the nearest residential receptor within 2.5 kilometers must always be reported.  If there is a worksite closer than the nearest residential receptor, both the worksite and residential receptor must be reported.  If there are no receptors within 2.5 kilometers from the facility boundary, simply report to the District the fact that the closest receptor is more than 2.5 kilometers away.  Please note that the District must be notified if any receptor distances change after submittal.]